Firmware Binary Comparison Tool

Design Document  ·  Feature Specification

| Item | Value |
| --- | --- |
| Document version | 1.0 |
| Software version | 1.0.0 — frozen baseline |
| Date | 16 October 2026 |
| Status | Frozen — feature complete for v1.0.0 |
| Prepared by | Vinaya Kumar |
| Verification | 84 automated tests passing; validated on STM32F411 and PIC32MX575 production firmware |

Revision history

| Version | Date | Description |
| --- | --- | --- |
| 1.0 | 16 Oct 2026 | First release of the design document for the frozen v1.0.0 baseline. |

# Contents

1. Introduction

1.1 Purpose · 1.2 Scope · 1.3 Intended audience · 1.4 Terms

2. Feature overview and requirements traceability

3. Architecture

3.1 Package structure · 3.2 Processing pipeline

4. Data model

4.1 Firmware image · 4.2 Address sets · 4.3 Configuration file

5. Comparison engine

5.1 Algorithm · 5.2 Gap-fill mode · 5.3 Results · 5.4 Performance

6. Region, block and expected-difference analysis

7. File format support

8. Function- and module-level change analysis

8.1 Symbol sources · 8.2 Matching and statuses · 8.3 Relocation-aware comparison · 8.4 Validation

9. Controller identification

10. Metadata, checksums and ELF comparison

11. Outputs: GUI, CLI and reports

11.1 GUI · 11.2 Command-line interface · 11.3 Reports

12. Engineering requirements compliance

13. Verification

14. Build, deployment and baseline

15. Known limitations and future work

# 1. Introduction

## 1.1 Purpose

This document describes the design and the complete feature set of the Firmware Binary Comparison Tool, a desktop and command-line utility for embedded firmware engineers. It compares two firmware images and shows exactly what changed, where it changed, which functions and modules were affected, and whether each change was expected. It is the reference for the frozen v1.0.0 baseline.

## 1.2 Scope

Comparison of raw binary (.bin), Intel HEX (.hex/.ihex/.ihx) and ELF (.elf/.axf/.out) firmware images, in any combination, by MCU memory address.

Region-based and expected-difference analysis, changed-block analysis and memory-map visualisation.

Function- and module-level change analysis from ELF or GNU ld / XC32 linker map files, with relocation-aware comparison that separates real edits from address shifts.

Heuristic controller identification, firmware metadata detection and checksums.

Engineering reports (HTML, CSV, JSON), a command-line interface for CI and a Tkinter GUI.

## 1.3 Intended audience

Embedded firmware engineers, test and release engineers, and maintainers of the tool.

## 1.4 Terms

| Term | Meaning |
| --- | --- |
| Address | MCU memory address a byte is programmed to (e.g. 0x08012A40). All analysis uses addresses. |
| File offset | Byte position inside the file on disk, always shown with a '+' prefix (+0x12A40). Intel HEX has no byte-level file offset (shown as n/a). |
| Base address | For raw .bin files: the MCU address of file offset 0. |
| LMA / VMA | Load (flash) address and virtual (run-time) address of ELF content. Images are compared at the LMA by default. |
| Region | Named, user-defined address range (Bootloader, Application, Calibration …). |
| Expected area | User-defined range that is allowed to change between builds (version, timestamp, CRC). Nothing is assumed to be expected unless configured. |
| Changed block | Contiguous run of differing bytes (optionally merged across small gaps). |
| RELOCATED | A function whose bytes differ only because call targets or pointers moved — a side effect, not an edit. |

# 2. Feature overview and requirements traceability

Table 1 maps every original requirement to its implementation. Features added during development are listed at the end.

Table 1: Requirements traceability

| # | Requirement | Implementation | Status |
| --- | --- | --- | --- |
| 1 | File selection, drag-and-drop, file information | Files tab: A/B cards with drop zones, SHA-256, format, address range, controller | Done |
| 2 | Byte-by-byte comparison statistics | core.binary_comparator + Summary tab / CLI | Done |
| 3 | Hex comparison viewer with navigation and search | gui.hex_viewer (virtualised) | Done |
| 4 | Memory region configuration, reusable files | core.region_manager + Configuration tab, JSON | Done |
| 5 | Region-based comparison | core.analysis + Regions tab / report | Done |
| 6 | Changed block analysis, sorting | core.block_detector + Changed Blocks tab | Done |
| 7 | Binary / memory-map visualisation | gui.memory_map + report SVG | Done |
| 8 | Difference filters | Hex viewer and block-table filters | Done |
| 9 | Expected / ignored areas, EXPECTED vs UNEXPECTED | core.analysis, verdict logic | Done |
| 10 | Firmware metadata detection (heuristic) | core.metadata_detector + Metadata tab | Done |
| 11 | CRC-32 / CRC-16 / SHA-256 | core.checksum + Checksums tab / --checksum | Done |
| 12 | ELF support: sections, symbols, build info | formats.elf_parser + core.elf_compare + ELF tab | Done |
| 13 | HTML report (10 sections) and CSV export | reports.html_report / reports.csv_report | Done |
| 14 | Command-line interface | cli.cli (firmware-diff), exit codes | Done |
| 15 | Python 3.11+, Tkinter, minimal dependencies, modular | Standard library only | Done |
| 16 | Engineering requirements (read-only, large files …) | See section 13 | Done |
| 17 | Engineering-tool UX | Tabbed GUI, header, tiles, tooltips | Done |
| + | Function / module level changes | core.symbols + core.feature_diff | Added |
| + | Relocation-aware comparison (ARM Thumb-2, MIPS32) | core.feature_diff | Added |
| + | Controller identification | core.mcu_detector | Added |
| + | PIC32 KSEG ↔ physical address mapping | formats.elf_parser, core.symbols | Added |
| + | Gap-fill mode (sparse HEX vs full BIN) | compare_images(fill=…) | Added |
| + | Recent comparisons, remembered folders, tooltips | gui.settings, gui.widgets | Added |

# 3. Architecture

The tool is a pure-Python 3.11 application with no mandatory third-party dependencies. It is split into layers so that the comparison engine can be used without the GUI — the CLI and the automated tests call the same API.

Figure 1: Layers and modules

## 3.1 Package structure

Table 2: Modules

| Package / module | Responsibility |
| --- | --- |
| core/binary_reader.py | Format detection (magic bytes, extension, content), SHA-256, loader dispatch, controller identification on load |
| core/image.py | FirmwareImage and Segment model; address ↔ file-offset mapping; reads, search, iteration |
| core/intervals.py | IntervalSet: sorted disjoint address ranges with O(log n) overlap queries |
| core/binary_comparator.py | Chunked hierarchical byte comparison over the union of address ranges |
| core/block_detector.py | Changed-block grouping (merge gap), per-block statistics, sorting |
| core/region_manager.py | Regions, expected areas, project configuration, validation, JSON I/O |
| core/analysis.py | Region results, expected/unexpected split, verdict |
| core/symbols.py | Symbol tables from ELF or GNU ld / XC32 map files, labels, aliases, LMA/VMA |
| core/feature_diff.py | Function / variable / module comparison, relocation-aware decoding |
| core/mcu_detector.py | Heuristic controller identification with evidence |
| core/metadata_detector.py | Heuristic version / date / time / git / product / board detection |
| core/checksum.py | CRC-32, CRC-16 variants, SHA-256 over files, images and ranges |
| core/elf_compare.py | ELF section and symbol comparison |
| core/memory_map_model.py | Data model for the map visualisation (GUI and report) |
| core/session.py | Orchestration: load → compare → analyse → metadata → ELF → features |
| formats/ | bin_parser (mmap), hex_parser (+writer), elf_parser, elf_builder (tests/examples) |
| reports/ | html_report, csv_report |
| cli/cli.py | firmware-diff command-line interface |
| gui/ | main_window, file_panel, comparison_view, features_view, hex_viewer, hex_model, memory_map, config_editor, details_view, widgets, theme, settings |

## 3.2 Processing pipeline

Figure 2: Comparison pipeline

The byte comparison runs once per file pair on a worker thread. Region, expected-area and symbol analysis are re-computed instantly when the configuration or symbol files change, without re-comparing bytes.

# 4. Data model

## 4.1 Firmware image

A FirmwareImage is a list of non-overlapping Segments. Each segment maps a range of MCU addresses to bytes and remembers its file offset when one exists:

Raw BIN: one segment at the configured base address; the file is memory-mapped read-only (mmap ACCESS_READ), so large images are not copied into memory.

Intel HEX: one segment per contiguous run of records; gaps are preserved, never filled.

ELF: one segment per PT_LOAD program header at its load address (LMA). VMA mode is optional. For PIC32 (MIPS32) KSEG0/KSEG1 addresses are converted to physical addresses so ELF, HEX and programmer addresses agree.

## 4.2 Address sets

Every set of addresses — file coverage, differences, expected areas, regions — is an IntervalSet of sorted half-open ranges with a prefix-sum index. This keeps memory proportional to the number of changed runs, not changed bytes, and answers 'how many changed bytes lie in [s, e)' in O(log n). Intersection, subtraction and union are linear merges.

## 4.3 Configuration file

Regions and expected areas are stored in one JSON project file (schema: firmware_binary_tool/config/schema.json). End addresses are inclusive; 'size' may replace 'end'; 'ignore' is an alias of 'expected'.

{
  "name": "STM32F4 XC-200 motor controller",
  "base_address": "0x08000000",
  "merge_gap": 0,
  "gap_fill": null,
  "regions": [
    { "name": "Bootloader",  "start": "0x08000000", "end": "0x08007FFF" },
    { "name": "Application", "start": "0x08008000", "end": "0x0803FFFF" }
  ],
  "expected": [
    { "name": "Version", "start": "0x08001000", "end": "0x0800101F" },
    { "name": "CRC",     "start": "0x0807FFFC", "end": "0x0807FFFF" }
  ]
}

Validation reports errors (start after end, invalid numbers, duplicate names, out-of-range values) and warnings (overlapping regions).

# 5. Comparison engine

## 5.1 Algorithm

The comparison runs over the union of both images' address ranges. Addresses present in both are compared byte by byte; addresses present in only one image are reported as ONLY A / ONLY B differences, so a size change is never silently ignored.

Data is processed in 64 KiB chunks. Identical chunks are skipped at memcmp speed; differing chunks are narrowed down hierarchically (4096 → 256 → 16 → 1 bytes) so only differing bytes are inspected individually.

Differences are accumulated as runs and stored in an IntervalSet; adjacent runs across chunk and segment boundaries are merged.

Progress reporting and cancellation are supported (worker thread in the GUI).

## 5.2 Gap-fill mode (optional)

A sparse Intel HEX file omits erased flash pages, so it differs from a full BIN dump of the same build. With gap fill (e.g. 0xFF) bytes present in only one image are compared against the fill value instead. The mode is off by default and its state is shown in every summary and report.

## 5.3 Results

Table 3: Summary statistics

| Statistic | Definition |
| --- | --- |
| Total compared | Bytes present in both images (plus one-sided bytes in gap-fill mode) |
| Different bytes / Changed % | All differing addresses / union of both images |
| First / last difference | Lowest / highest differing address, with file offsets of A and B |
| Changed blocks / largest block | Contiguous runs (merge gap optional) |
| Only in A / only in B | Bytes with data in one image only |
| Expected / unexpected changes | Differences inside / outside configured expected areas |
| Verdict | IDENTICAL, EXPECTED CHANGES ONLY or UNEXPECTED CHANGES |

## 5.4 Performance (measured)

Table 4: Performance

| Case | Time |
| --- | --- |
| Byte comparison, 64 MB image with 2,000 changed areas | ≈ 0.1 s |
| Full session for the same images (hashing, metadata scan) | ≈ 2 s |
| Every other byte differs in 4 MB (2 million blocks, pathological) | ≈ 9 s |
| Real STM32F411 / PIC32MX575 production images | < 0.1 s |

# 6. Region, block and expected-difference analysis

Regions: for every region the tool reports size, changed bytes, changed %, expected and unexpected bytes, coverage of A and B, and a status: SAME, CHANGED, EXPECTED CHANGE or NOT PRESENT. Changes outside all regions are reported in their own row.

Changed blocks: start, end, size, changed bytes, expected/unexpected bytes, classification (EXPECTED / UNEXPECTED / MIXED), regions, kind (modified / only in A / only in B), file offsets and the containing function. Sorting by address, size, changed bytes or region; filters by class, region and minimum size.

Expected areas: a difference is EXPECTED only inside an area the user configured. Each area reports CHANGED (EXPECTED), UNCHANGED or NOT PRESENT.

Memory map: region tree, linear region bar, change strip and a clickable block grid over a compressed address axis (gaps without data are omitted and marked).

Figure 3: Regions tab — per-region statistics with expected/unexpected split

Figure 4: Memory map — region layout, change strip and block grid

# 7. File format support

Table 5: Supported formats

| Format | Support | Validation / errors |
| --- | --- | --- |
| Raw BIN | Memory-mapped read-only; base address per file or from configuration | Empty file → warning; never modified |
| Intel HEX | Records 00–05, extended segment / linear addressing, start addresses; HEX writer for examples | Line-level errors for bad syntax, length, checksum, type; overlapping records rejected; missing EOF warned |
| ELF | ELF32/64, little/big endian; PT_LOAD segments at LMA (or VMA); sections, symbols, .comment, GNU build-id; PIC32 KSEG→physical | Bounds-checked tables; truncated or invalid files give a clear error |
| GNU ld / XC32 map | Input sections, symbols (incl. COMMON), modules, load addresses, linker labels; PIC32 detection | Rejects files without a memory-map section |

Format detection: ELF by magic, Intel HEX by extension or text records, otherwise raw binary. The format can be forced per file.

# 8. Function- and module-level change analysis

A binary does not know about 'features'. The closest reliable proxy is the symbol information of each build: functions, variables and the source module they come from.

## 8.1 Symbol sources

ELF: symbol table, Thumb bit cleared, LMA computed from program headers, source file of local symbols from STT_FILE entries, size-0 linker labels collected separately.

Linker map: GNU ld and XC32 map files; gives the object file (module) of every symbol and the load address of initialised data.

## 8.2 Matching and statuses

Aliases (several names at one address) are grouped into one symbol.

Symbols are matched by name and module; compiler-generated names that change between builds (C.55.31329, counter.1234) are paired by identical content or unique candidates.

Each side's bytes are read at that side's own address and compared.

Table 6: Function statuses

| Status | Meaning | Counted as edit |
| --- | --- | --- |
| MODIFIED | Content changed | Yes |
| ADDED / REMOVED | Only in the new / old build | Yes |
| LIKELY RELOCATED | Only address references differ, some to unnamed data — listed for review | Review |
| RELOCATED | Only call targets / pointers changed because referenced code or data moved | No |
| MOVED | Identical content at a new address | No |
| UNCHANGED / NOT IN IMAGE | Identical / RAM-only (.bss) | No |

## 8.3 Relocation-aware comparison

Removing or adding code shifts everything linked after it, which changes the bytes of many unedited functions. Every differing branch or pointer is therefore decoded in both builds and resolved to a build-independent key (symbol + offset, linker label, identical pointed-to content, or position relative to neighbouring symbols):

ARM / Cortex-M: Thumb-2 BL / BLX / B.W, ARM B / BL, literal-pool pointers.

MIPS32 / PIC32: J / JAL, LUI + ADDIU / load / store address pairs with register tracking (including branch delay slots), $gp-relative small-data accesses.

## 8.4 Validation on production firmware

Table 7: Real-firmware validation

| Firmware | Change | Result |
| --- | --- | --- |
| STM32F411 analysis board | Watchdog commented out | 2 modified (peripheralInit, main), 6 removed (iwdgInit, HAL_IWDG_Init, iwdgRefresh, HAL_IWDG_Refresh, iwdgInstance, iwdgHandle); 762 side effects separated. Before relocation analysis: 350 'modified'. |
| PIC32MX575F512L system controller | Release IR44 → IR45 | Modbus stack added, Chinese language data removed, 91 size-changed functions; unexplained bytes reduced from 420,619 to ~21–36 k after KSEG mapping. |

Figure 5: Changed Functions — exactly the watchdog change in a 900-symbol firmware

# 9. Controller identification

The target controller is identified heuristically and every result lists its evidence with a confidence.

Table 8: Controller evidence

| Evidence | Example | Gives | Confidence |
| --- | --- | --- | --- |
| ELF debug info / strings | stm32f411xe.h, startup_stm32f411retx.s, p32mx575f512l.h | Exact part | High |
| Linker map | 32MX575F512L (XC32) | Exact part | High |
| Compiler options | -mcpu=cortex-m4 -mfpu=fpv4-sp-d16 | Core, FPU | High |
| Memory layout | 0x08000000 flash; 0x1D000000 + 0x1FC00000 | Family | Medium / low |
| PIC32 configuration words | 0x1FC02FF0 / 0x1FC00BF0 / 0x1FC0FFxx | MX3-7 / MX1-2 / MZ | Medium |
| Cortex-M vector table | SP 0x20020000, 102 vectors | RAM ≥ 128 KB, 86 IRQs | Medium |
| Image headers | ESP 0xE9 + chip id, AVR JMP table | ESP32 variant / AVR | Medium |

Part numbers are decoded (STM32F411RET → 512 KB flash, 64 pins, LQFP; PIC32MX575F512L → 512 KB, 100 pins). An application starting inside flash is flagged as likely behind a bootloader. A warning is shown when A and B target conflicting controllers; an exact part within a family-level result is treated as compatible.

Figure 6: Files tab — PIC32 identification refined by the linker map

# 10. Metadata, checksums and ELF comparison

Metadata (heuristic): printable strings are matched against patterns for firmware version, build date / time (__DATE__ / ISO), git commit, product and board name, each with a confidence and address. The UI and reports state that detection is not guaranteed.

Checksums: CRC-32 (ISO-HDLC), CRC-16/CCITT-FALSE, XMODEM, MODBUS, ARC and SHA-256 over the file, the image data, a region or a custom range with gap fill. The tool never assumes the firmware's own CRC algorithm.

ELF comparison: sections (address, size, differing bytes, status, notes), build information (compiler, build-id) and symbol changes (added / removed / moved / resized).

Figure 7: Metadata tab with heuristic disclaimer

# 11. Outputs: GUI, CLI and reports

## 11.1 Graphical user interface

Table 9: GUI tabs

| Tab | Content |
| --- | --- |
| 1 Files | A/B cards (drop zone, format, base address, file details, controller), Compare / Swap, symbol files, configuration, controller evidence |
| 2 Configuration | Regions, expected areas, base address, merge gap, gap fill; load/save/import |
| Summary | Verdict banner, number tiles, details, 'What changed' function list |
| Regions / Changed Blocks | Per-region and per-block tables with filters, CSV export |
| Changed Functions | Modules, functions, real / likely relocated / side-effect filters |
| Memory Map | Region tree, region bar, change strip, clickable block grid |
| Metadata / Checksums / ELF | Heuristic metadata, checksum calculator, ELF sections and symbols |
| Hex Comparison (window) | Virtualised side-by-side or 1-byte table view; first/prev/next/last difference; go to address or file offset; hex search; filters; status bar with offsets, region, block and function |

Usability: header with file names and verdict badge, colour coding (A blue, B purple), tooltips, helpful empty states, remembered folders and window size, File → Recent comparisons, Quick start and keyboard shortcuts, high-DPI awareness on Windows.

Figure 8: Summary tab

Figure 9: Hex comparison viewer

## 11.2 Command-line interface

Table 10: Main CLI options

| Option | Purpose |
| --- | --- |
| --regions / --ignore | Project / region file; extra expected areas |
| --report / --csv / --json | HTML report, changed-block CSV, JSON summary ('-' = stdout) |
| --base / --base-a / --base-b | Base address for raw .bin files |
| --format-a / --format-b / --elf-vaddr | Force format; use ELF VMA |
| --fill / --merge-gap | Gap-fill value; block merge gap |
| --symbols-a / --symbols-b | ELF or map file per build (function-level analysis) |
| --functions / --show-relocated / --functions-csv | Function listing, side effects, CSV |
| --checksum ALG | CRC-32, CRC-16 variants, SHA-256 |
| --controller-details / --no-metadata / -q | Evidence output, skip metadata, verdict only |

Exit codes: 0 = identical or only expected changes, 1 = unexpected changes, 2 = error — suitable as a CI gate.

Figure 10: CLI output

## 11.3 Reports

The HTML report is self-contained (no external resources) and contains: firmware A and B information (including controller), SHA-256 hashes, overall summary with memory map, region comparison, changed blocks with function names, expected changes, unexpected changes, function and module changes (with side effects and likely-relocated sections), metadata differences, ELF comparison and the comparison timestamp. CSV exports cover changed blocks and function changes; JSON provides a machine-readable summary.

# 12. Engineering requirements compliance

Table 11: Engineering requirements

| Requirement | How it is met |
| --- | --- |
| Never modify original files | Files opened read-only; BIN mmap with ACCESS_READ; test verifies hash and modification time are unchanged |
| Large images | mmap, 64 KiB chunked hierarchical comparison, run-based storage, virtualised hex viewer, capped tables with CSV for full lists |
| File offset vs MCU address | Separate columns everywhere; offsets prefixed '+', n/a for Intel HEX |
| Never silently ignore differences | One-sided bytes counted, outside-region changes listed, unattributed bytes reported, gap fill opt-in and always displayed |
| Clear errors | Typed exceptions with line numbers / offsets; GUI dialogs and status bar; CLI exit code 2 |
| Validate memory regions | Range, number, duplicate-name checks; overlap warnings |
| Different sizes, empty and malformed files | ONLY A / ONLY B, empty-file warnings, parser validation |
| Unit tests for the engine | 84 tests (section 13) |

# 13. Verification

Table 12: Automated tests (84, all passing)

| Test module | Tests | Coverage |
| --- | --- | --- |
| test_intervals | 5 | Interval algebra checked against Python sets on random data |
| test_comparator | 12 | Naive byte-by-byte reference on random data, chunk boundaries, sizes, empty, sparse, gap fill, progress / cancel |
| test_blocks_regions | 15 | Merge gap, blocks across regions, expected vs unexpected, config validation |
| test_formats | 17 | BIN, HEX (malformed, overlapping, extended addressing), ELF (truncated, VMA/LMA), files never modified |
| test_checksum_metadata | 7 | CRC catalogue check values, chunking, metadata heuristics |
| test_features | 11 | Map parsing, function statuses, real Thumb-2 and MIPS code, aliases, labels, renamed symbols, PIC32 KSEG |
| test_mcu_detector | 8 | Part decoding, vector tables, PIC32 layout, mismatch rules |
| test_reports_cli | 8 | HTML / CSV / JSON content, exit codes, hex row model |
| test_gui_smoke | 1 | Drives the real GUI: load, compare, tabs, hex navigation, filters, search |

In addition the tool was validated manually on two production firmware pairs (section 8.4) and on the deterministic example builds in examples/.

# 14. Build, deployment and baseline

Runtime: Python 3.11+ standard library; optional tkinterdnd2 for drag-and-drop.

Run: python firmware_compare_gui.py (GUI), python firmware_diff.py (CLI), or pip install . for the firmware-diff / firmware-diff-gui commands.

Standalone executables: build_windows.bat / build_linux.sh (PyInstaller).

Frozen baseline: docs/FREEZE_v1.0.0.md lists the SHA-256 of every source, test and build file.

Per-user settings: %APPDATA%\FirmwareBinaryTool\settings.json (Windows) or ~/.config/firmware_binary_tool (Linux).

# 15. Known limitations and future work

Instruction decoding covers ARM/Thumb-2 and MIPS32; MOVW/MOVT pairs, microMIPS/MIPS16e and other architectures (e.g. RISC-V) are not decoded — address-only changes there may appear as MODIFIED.

Map parsing supports GNU ld and XC32; IAR and Keil map files are not supported (use their ELF output).

Metadata and controller identification are heuristic and labelled as such.

Intel HEX and ELF files are parsed into memory (raw BIN files are memory-mapped).

Motorola S-record is not supported.

Same-size MODIFIED functions can be side effects of header changes (e.g. struct layout) or compiler scheduling; the binary shows that code changed, not which source line.

Candidate future work: S-record, IAR/Keil maps, RISC-V decoding, git integration, batch comparison.